DIGITAL LOGIC DESIGN

Number System Conversion

Practice Guide: Lectures 1 & 2

Name: ______________________________      Date: ______________      Score: ______ / 101

How to use this guide 1. Read the method box at the top of each part, then study the worked example (taken from the lectures). 2. Solve the problems by hand in the answer column, with no calculator. Use the margin for working. 3. Check your work against the answer key on page 10. If you get one wrong, redo it and find your mistake. 4. Do the parts in order. Each one builds on the one before it.

## Roadmap

| Part | Skill | Page | Questions |
| --- | --- | --- | --- |
| 1 | Any base to decimal (integers) | 2 | 14 |
| 2 | Decimal to binary, octal, hexadecimal (integers) | 3 | 15 |
| 3 | Decimal fractions to other bases | 4 | 17 |
| 4 | Fractions in other bases to decimal | 5 | 13 |
| 5 | Binary ↔ Octal | 6 | 10 |
| 6 | Binary ↔ Hexadecimal | 7 | 10 |
| 7 | Octal ↔ Hexadecimal and fill-in table | 8 | 12 |
| 8 | Mixed review, error spotting and self-check | 9 | 10 |
|  | Answer key | 10 |  |

## Quick reference: 0 to 15 in every base

| Decimal; Binary; Octal; Hex. 0; 0000; 0; 0. 1; 0001; 1; 1. 2; 0010; 2; 2. 3; 0011; 3; 3. 4; 0100; 4; 4. 5; 0101; 5; 5. 6; 0110; 6; 6. 7; 0111; 7; 7. | Decimal; Binary; Octal; Hex. 8; 1000; 10; 8. 9; 1001; 11; 9. 10; 1010; 12; A. 11; 1011; 13; B. 12; 1100; 14; C. 13; 1101; 15; D. 14; 1110; 16; E. 15; 1111; 17; F. |
| --- | --- |

# Part 1: Any Base → Decimal (Integers)

Method: the polynomial method Value = Σ (digit × baseposition). Number the positions from the right, starting at 0. Step 1: write each digit with its position. Step 2: multiply the digit by baseposition. Step 3: add the results. Powers to know: 2: 1, 2, 4, 8, 16, 32, 64, 128, 256 ¦ 8: 1, 8, 64, 512, 4096 ¦ 16: 1, 16, 256, 4096. Hex letters: A=10, B=11, C=12, D=13, E=14, F=15.

## Worked examples (from Lecture 1)

(11010)2 = 1×16 + 1×8 + 0×4 + 1×2 + 0×1 = (26)10

(726)8 = 7×64 + 2×8 + 6×1 = 448 + 16 + 6 = (470)10

(27FA)16 = 2×4096 + 7×256 + 15×16 + 10×1 = 8192 + 1792 + 240 + 10 = (10234)10

## Practice: convert each number to decimal

| # | Convert | Your answer |
| --- | --- | --- |
| 1 | (1101)2 → base 10 |  |
| 2 | (10110)2 → base 10 |  |
| 3 | (111001)2 → base 10 |  |
| 4 | (1010101)2 → base 10 |  |
| 5 | (11111111)2 → base 10 |  |
| 6 | (61)8 → base 10 |  |
| 7 | (157)8 → base 10 |  |
| 8 | (405)8 → base 10 |  |
| 9 | (3042)8 → base 10 |  |
| 10 | (2F)16 → base 10 |  |
| 11 | (B8)16 → base 10 |  |
| 12 | (3C5)16 → base 10 |  |
| 13 | (1A3F)16 → base 10 |  |
| 14 | (FF)16 → base 10 |  |

# Part 2: Decimal → Binary, Octal, Hexadecimal

Method: repeated division Divide the number by the new base. Write the quotient and the remainder. Keep dividing the quotient until it is 0. Read the remainders from bottom to top: the last remainder is the MSB and the first is the LSB. For hexadecimal, remainders 10–15 are written as A–F.

## Worked examples (from Lecture 2)

(29)10:  29÷2 = 14 r 1 (LSB) · 14÷2 = 7 r 0 · 7÷2 = 3 r 1 · 3÷2 = 1 r 1 · 1÷2 = 0 r 1 (MSB)   →   (11101)2

(1341)10 = (10100111101)2  =  (2475)8  =  (53D)16   (13 = D)

## Practice

| # | Convert | Your answer |
| --- | --- | --- |
| 1 | (13)10 → binary |  |
| 2 | (45)10 → binary |  |
| 3 | (92)10 → binary |  |
| 4 | (156)10 → binary |  |
| 5 | (255)10 → binary |  |
| 6 | (64)10 → octal |  |
| 7 | (100)10 → octal |  |
| 8 | (255)10 → octal |  |
| 9 | (511)10 → octal |  |
| 10 | (1000)10 → octal |  |
| 11 | (255)10 → hexadecimal |  |
| 12 | (500)10 → hexadecimal |  |
| 13 | (1000)10 → hexadecimal |  |
| 14 | (4095)10 → hexadecimal |  |
| 15 | (2026)10 → hexadecimal |  |

# Part 3: Decimal Fractions → Binary, Octal, Hexadecimal

Method: repeated multiplication Multiply only the fractional part by the new base. The whole-number part of the result is the next digit. Keep the new fractional part and repeat. Stop when it reaches 0 or when you have enough digits. Read the digits from top to bottom (first digit = MSB). For a mixed number such as 374.37, convert the integer part by division and the fraction part by multiplication, then join them with the point.

## Worked examples (from Lecture 2)

(0.188)10 × 2 → 0.376 (0), 0.752 (0), 1.504 (1), 1.008 (1), 0.016 (0)  →  (.00110)2 (5 digits)

(0.23)10 × 8 → 1.84 (1), 6.72 (6), 5.76 (5)  →  (0.165)8

(374.37)10 → integer 176, fraction 0.5EB8  →  (176.5EB8)16

## Practice

| # | Convert | Your answer |
| --- | --- | --- |
| 1 | (0.625)10 → binary |  |
| 2 | (0.375)10 → binary |  |
| 3 | (0.8125)10 → binary |  |
| 4 | (0.1875)10 → binary |  |
| 5 | (0.3)10 → binary (5 digits) |  |
| 6 | (0.1)10 → binary (6 digits) |  |
| 7 | (0.5)10 → octal |  |
| 8 | (0.125)10 → octal |  |
| 9 | (0.4)10 → octal (3 digits) |  |
| 10 | (0.6)10 → octal (3 digits) |  |
| 11 | (0.75)10 → hexadecimal |  |
| 12 | (0.3125)10 → hexadecimal |  |
| 13 | (0.9)10 → hexadecimal (3 digits) |  |
| 14 | (0.2)10 → hexadecimal (4 digits) |  |
| 15 | (13.625)10 → binary |  |
| 16 | (25.75)10 → octal |  |
| 17 | (42.5)10 → hexadecimal |  |

# Part 4: Fractions in Other Bases → Decimal

Method: negative powers after the point Digits to the right of the point have negative positions: the first is base−1, the next base−2, and so on. Add the integer part and the fraction part. Useful values: 2−1=0.5, 2−2=0.25, 2−3=0.125, 2−4=0.0625 ¦ 8−1=0.125, 8−2=0.015625 ¦ 16−1=0.0625, 16−2=0.00390625

## Worked examples (from Lecture 2)

(234.14)8 = 2×64 + 3×8 + 4×1 + 1×0.125 + 4×0.015625 = 156 + 0.125 + 0.0625 = (156.1875)10

(4B3.3)16 = 4×256 + 11×16 + 3×1 + 3×0.0625 = 1024 + 176 + 3 + 0.1875 = (1203.1875)10

## Practice: convert to decimal

| # | Convert | Your answer |
| --- | --- | --- |
| 1 | (101.101)2 → decimal |  |
| 2 | (11.011)2 → decimal |  |
| 3 | (1101.1)2 → decimal |  |
| 4 | (1010.0101)2 → decimal |  |
| 5 | (0.111)2 → decimal |  |
| 6 | (17.4)8 → decimal |  |
| 7 | (62.2)8 → decimal |  |
| 8 | (5.34)8 → decimal |  |
| 9 | (100.01)8 → decimal |  |
| 10 | (A.8)16 → decimal |  |
| 11 | (2F.C)16 → decimal |  |
| 12 | (1E.4)16 → decimal |  |
| 13 | (3.A8)16 → decimal |  |

# Part 5: Binary ↔ Octal

Method: groups of 3 bits Binary → Octal: split the bits into groups of 3. Start at the point and work outward: the integer part from the right, the fraction part from the left. Pad with zeros if a group is short. Convert each group (000–111 = 0–7). Octal → Binary: replace each octal digit with its 3-bit group. Drop leading zeros at the end. 3-bit groups: 0=000 1=001 2=010 3=011 4=100 5=101 6=110 7=111

## Worked examples (from Lecture 2)

(10101)2 → 010 | 101 → 2 5 = (25)8        (25)8 → 010 | 101 = (10101)2

## Practice

| # | Convert | Your answer |
| --- | --- | --- |
| 1 | (101110)2 → octal |  |
| 2 | (11010111)2 → octal |  |
| 3 | (1111001)2 → octal |  |
| 4 | (100101011)2 → octal |  |
| 5 | (110101.101)2 → octal |  |
| 6 | (47)8 → binary |  |
| 7 | (725)8 → binary |  |
| 8 | (1604)8 → binary |  |
| 9 | (3.52)8 → binary |  |
| 10 | (0.6)8 → binary |  |

# Part 6: Binary ↔ Hexadecimal

Method: groups of 4 bits Binary → Hex: split the bits into groups of 4, working outward from the point (integer part from the right, fraction part from the left). Pad with zeros. Convert each group (0000–1111 = 0–F). Hex → Binary: replace each hex digit with its 4-bit group. Memory aid: 8-4-2-1. Example: 1011 = 8+0+2+1 = 11 = B. The full 0–15 table is on page 1.

## Worked examples (from Lecture 2)

(10101)2 → 0001 | 0101 → 1 5 = (15)16        (15)16 → 0001 | 0101 = (10101)2

## Practice

| # | Convert | Your answer |
| --- | --- | --- |
| 1 | (11010110)2 → hexadecimal |  |
| 2 | (1011111001)2 → hexadecimal |  |
| 3 | (111100001111)2 → hexadecimal |  |
| 4 | (1101.1011)2 → hexadecimal |  |
| 5 | (101101.011)2 → hexadecimal |  |
| 6 | (3A)16 → binary |  |
| 7 | (7F2)16 → binary |  |
| 8 | (B09C)16 → binary |  |
| 9 | (5.E8)16 → binary |  |
| 10 | (2D.4)16 → binary |  |

# Part 7: Octal ↔ Hexadecimal and Fill-in Table

Method: use binary as the bridge There is no direct shortcut between octal and hex. Convert the first number to binary (3 bits per octal digit or 4 bits per hex digit). Then regroup the same bits (4 bits for hex or 3 bits for octal) and convert. Example: (17)8 → 001 111 → 0000 1111 → (0F)16 = (F)16

## A. Convert through binary

| # | Convert | Your answer |
| --- | --- | --- |
| 1 | (275)8 → hexadecimal |  |
| 2 | (1234)8 → hexadecimal |  |
| 3 | (777)8 → hexadecimal |  |
| 4 | (3E)16 → octal |  |
| 5 | (7C1)16 → octal |  |
| 6 | (2F9)16 → octal |  |

## B. Complete the table (one value is given in each row)

| Row | Decimal | Binary | Octal | Hexadecimal |
| --- | --- | --- | --- | --- |
| 1 | 21710 |  |  |  |
| 2 |  | 1011010112 |  |  |
| 3 |  |  | 3458 |  |
| 4 |  |  |  | 9D16 |
| 5 | 102310 |  |  |  |
| 6 | 5010 |  |  |  |

# Part 8: Mixed Review, Error Spotting and Self-Check

## A. Spot the mistake

| # | Convert | Your answer |
| --- | --- | --- |
| 1 | A student converts (29)10 to binary and reads the remainders from top to bottom, getting (10111)2. What went wrong? What is the correct answer? | Mistake / correct answer: |
| 2 | A student converts (1101)2 to octal by grouping from the left: 11 ¦ 01 → (31)8. What went wrong? What is the correct answer? | Mistake / correct answer: |
| 3 | A student converts (0.375)10 to binary by dividing by 2 and gets (0.1875)10. Which operation should be used, and what is the correct binary result? | Mistake / correct answer: |
| 4 | A student says (2F)16 = 2×16 + 6 = 38 because "F is 6". What is the correct value? | Mistake / correct answer: |

## B. Multi-base challenge

| # | Convert | Your answer |
| --- | --- | --- |
| 5 | (1011011)2 | Decimal: ____ Octal: ____ Hex: ____ |
| 6 | (3B)16 | Decimal: ____ Binary: ____________ |
| 7 | (400)10 | Binary: ______ Octal: ____ Hex: ____ |
| 8 | (52.625)10 | Binary: ______ Octal: ____ Hex: ____ |
| 9 | (111.011)2 | Decimal: ____ Octal: ____ Hex: ____ |
| 10 | (7A.8)16 | Decimal: ____ Binary: ____________ |

## C. Self-check: tick the skills you can do without notes

- Convert any base to decimal using positional powers (Part 1 and 4)

- Convert decimal integers to binary, octal and hex by repeated division (Part 2)

- Convert decimal fractions by repeated multiplication (Part 3)

- Convert between binary and octal or hex by grouping 3 or 4 bits (Parts 5 and 6)

- Convert octal ↔ hex through binary (Part 7)

# Answer Key

Check your answers once you have finished each part. Subscripts show the base. A fraction answer with a digit limit is rounded by cutting off, not by rounding.

Part 1 → decimal:  1) 13    2) 22    3) 57    4) 85    5) 255    6) 49    7) 111    8) 261    9) 1570    10) 47    11) 184    12) 965    13) 6719    14) 255

Part 2 → binary (1–5), octal (6–10), hex (11–15):  1) 11012    2) 1011012    3) 10111002    4) 100111002    5) 111111112    6) 1008    7) 1448    8) 3778    9) 7778    10) 17508    11) FF16    12) 1F416    13) 3E816    14) FFF16    15) 7EA16

Part 3 → binary (1–6), octal (7–10), hex (11–14), mixed (15–17):  1) 0.1012    2) 0.0112    3) 0.11012    4) 0.00112    5) 0.010012    6) 0.0001102    7) 0.48    8) 0.18    9) 0.3148    10) 0.4638    11) 0.C16    12) 0.516    13) 0.E6616    14) 0.333316    15) 1101.1012    16) 31.68    17) 2A.816

Part 4 → decimal:  1) 5.625    2) 3.375    3) 13.5    4) 10.3125    5) 0.875    6) 15.5    7) 50.25    8) 5.4375    9) 64.015625    10) 10.5    11) 47.75    12) 30.25    13) 3.65625

Part 5 → octal (1–5), binary (6–10):  1) 568    2) 3278    3) 1718    4) 4538    5) 65.58    6) 1001112    7) 1110101012    8) 11100001002    9) 11.1010102    10) 0.1102

Part 6 → hex (1–5), binary (6–10):  1) D616    2) 2F916    3) F0F16    4) D.B16    5) 2D.616    6) 1110102    7) 111111100102    8) 10110000100111002    9) 101.111010002    10) 101101.01002

Part 7A → octal ↔ hex:  1) BD16    2) 29C16    3) 1FF16    4) 768    5) 37018    6) 13718

Part 7B (table):  1) 21710 = 110110012 = 3318 = D916     2) 36310 = 1011010112 = 5538 = 16B16     3) 22910 = 111001012 = 3458 = E516     4) 15710 = 100111012 = 2358 = 9D16     5) 102310 = 11111111112 = 17778 = 3FF16     6) 5010 = 1100102 = 628 = 3216

Part 8A (errors):  1) Read the remainders bottom to top: 111012.   2) Group from the right: 158.   3) Multiply by 2: 0.0112.   4) F = 15, so 2×16 + 15 = 47.

Part 8B (challenge):  5) Dec 9110, Oct 1338, Hex 5B16     6) Dec 5910, Bin 1110112     7) Bin 1100100002, Oct 6208, Hex 19016     8) Bin 110100.1012, Oct 64.58, Hex 34.A16     9) Dec 7.37510, Oct 7.38, Hex 7.616     10) Dec 122.510, Bin 1111010.10002

Scoring guide Count your correct answers across Parts 1–8. There are 101 questions in total (each table row and each challenge question counts as 1). 90% or more: excellent, you are ready for the exam. 70–89%: review the parts where you lost marks. Below 70%: re-read the method boxes and redo Parts 1–3 before moving on.